# Capacity-feasible CVRP clustering on an analog neutral-atom register (Track A)

Pasqal challenge — team DuQuantum — repository qhack

2026-10-25

# 1. Model

Bottom line. The register decides which customers share a vehicle; each group is then routed exactly. One atom per (customer, vehicle) pair gives 8 / 30 / 60 atoms on tiers S / M / L. Raw quantum results are strong on S, poor on M, and almost never feasible on L. Two additions sharpen that conclusion: a held-out capacity test improves raw feasibility with a stronger load-balance weight, while a depot-aware classical objective greatly reduces route loss on L but has not yet been embedded. Classical repair is reported separately and gives no quantum advantage over matched random samples.

Why cluster-first. Encoding the full route (customer, vehicle, position) needs about N2K atoms: 32 / 300 / 900 for S / M / L, against 80 available (DERIVATIONS Q1). Deciding only the grouping needs NK. The cost is a decomposition gap: the best grouping under our objective is not always the grouping of the shortest routes. It is 0 % on 5 of 11 instances and up to 21.7 % on the rest (§5, decomposition table).

Variables. N customers with demands di; K identical vehicles of capacity Q; dij the distance between customers i and j. The bit xik=1 means customer i rides on vehicle k. A valid grouping has exactly one 1 in every customer’s row (“one-hot”).

Objective. We want groups whose members are close to each other, so we add up the distances between every pair of customers that share a vehicle:  It costs us the decomposition gap above, and it is additive over groups, which is what lets an exact dynamic programme certify the model optimum on every tier (cvrp/exact.py).

Constraint 1 — every customer on exactly one vehicle. Enforced by a squared penalty with weight A:  Expanded, it puts the same linear term −A on every atom and a coupling +2A between the K atoms of one customer (a “clique”). It costs nothing in atoms; it costs energy scale: A must exceed every other term (below), and it is the largest coupling the register has to realise.

Constraint 2 — vehicle load Lk=i​dixik at most Q. We use unbalanced penalization (Montañez-Barrera et al. 2024), k​−λ1Q−Lk+λ2Q−Lk2, because it needs no slack atoms. On one-hot strings its linear part is the same constant for every valid grouping, so it drops out together with λ1 (the “one-hot identity”, DERIVATIONS §4); what survives is a load-balance term  The balance term alone has a feasible minimiser for K=2 whenever a feasible split exists; adding the distance objective removes that guarantee, and no such guarantee exists for K≥3. We therefore score feasible decodes only.

Three objectives must be separated. The constrained problem minimises Eq. (1) subject to both constraints. The QUBO sent to the embedder is, up to the offset AN,  The device realises Edevx=−δfa​xa+a<b​Uabxaxb. Positive couplings and uniform linear terms make the QUBO compatible in sign, but planar Uab=C6/rab6 still approximates it.

Weights. A=maxij≠i​dij+2λ2didj+1, which provably makes every minimiser one-hot (DERIVATIONS §4). λ2=0.15 R/2Q with R=maxij​dij, from a 40-seed brute-force scan. Brute force on the five S instances: the minimiser is feasible and optimal on 5/5; A survives one halving and fails at A/4; λ2 is not binding on any S instance.

# 2. Variable accounting (brief Derive 4.1)

| Encoding | count | S | M | L | fits 80? |
| --- | --- | --- | --- | --- | --- |
| assignment variables only | NK | 8 | 30 | 60 | yes |
| + binary slack (textbook) | NK+K⌈log2Q+1⌉ | 14 | 45 | 80 | yes (L = cap) |
| ours (onehot-ub) | NK | 8 | 30 | 60 | yes |

Tiers: S N=4,K=2,Q=7; M N=10,K=3,Q=20; L N=15,K=4,Q=30 (scripts/variable_count.py). We save 6 / 15 / 20 atoms over the textbook encoding and never discard a readout bit.

Instances. A seeded generator first draws a balanced assignment and then draws demands within each vehicle’s remaining capacity; that assignment is the shipped certificate and the validator rechecks it. Locations are uniform in 0,1002 and the depot is fixed at 50,50. Across S/M/L, ρ ranges are 0.643–0.857 / 0.700–0.950 / 0.600–0.708 and dmax/Q ranges are 0.429–0.571 / 0.400 / 0.300–0.333.

# 3. Embedding (DERIVATIONS §5)

Bottom line. A generic least-squares placement cannot realise the rook’s graph. We therefore fit device energies on one-hot strings. On the five tested S instances the realised register preserves the optimal clustering; it does not reproduce every model energy exactly. On M/L it fails to preserve the ordering of groupings.

Why generic placement fails. The K atoms of one customer must sit 5 µm apart (the clique). Any atom of another customer is then at nearly the same distance from all K of them, so the couplings that should be zero (different customer and different vehicle) come out as large as the wanted ones. Measured: relative coupling error 0.95, defined as ∥U/α−J∥F/∥J∥F over all pairs (lower is better; cvrp/embed.py), and on S_seed04 the register’s ground state was an infeasible string.

Ask for less. On a one-hot string each pair of customers has exactly one active coupling, so only the difference same-vehicle minus cross-vehicle has to match the model (DERIVATIONS 5.7b); the cross couplings only need to be constant. Each customer becomes a rigid 5 µm K-gon, which realises the clique exactly with α=C6/5 μm6/2A; the centres and orientations are fitted (L-BFGS, 6 seeded starts) so that the device energies of all one-hot strings match α EQUBO, with single-flip defects forced above their parents and the final detuning δf as a fit parameter. Registers are committed in embeddings/; every run checks the realised register (is its own minimiser feasible and optimal? smallest defect gap). Tier S: 5/5 yes, coupling error 0.21–0.32.

Figure 1. Left: S_seed00 register and modal feasible shot. Middle: target and realised couplings; the one-hot-subspace error is 0.382. Right: tier-S pulse scan; 0.75 times maximum amplitude at 5800 ns performs best among tested settings.

An isolated-polygon bound. Ignoring other polygons, one atom of a K-gon excited costs −δf; two excited cost −2δf+Umin, where Umin is the coupling of the weakest pair inside the polygon. One one excitation is preferred only if  For K≤3 every pair is the 5 µm side, Umin=55.1 rad/µs; the S fits land at δf=35–38 rad/µs (0.64–0.68 Umin). On M the fit is ill-conditioned and ran to the device maximum, 125.66 rad/µs =2.28 Umin, which favours two vehicles per customer; the embedder now enforces Eq. (4) (fallback 0.65 Umin). For K=4 a 5 µm square has 7.07 µm diagonals, and 1/r6 turns that 2 into a factor 8: Umin=6.9 rad/µs, so the L registers run at δf=4.5 rad/µs. Four points in a plane are never equidistant, so a planar one-hot clique for K≥4 is never uniform. Interactions with occupied atoms in other polygons also enter the excitation cost, so Eq. (5) is useful but not a sufficient global guarantee.

# 4. Pulse (DERIVATIONS §6)

Chosen: 5800 ns (device maximum minus 200 ns), Ω=0.75 Ωmax, fitted δf. Three-point Ω ramp (15 % up, 70 % plateau, 15 % down) and a linear detuning sweep during the plateau from δ0=−minmaxδf,2Ωmax,δmax to δf. Starting far below resonance puts every atom in |g⟩; the positive final detuning and defect-gap fit target one excitation per customer. Every sequence is built with Pulser on AnalogDevice and validated against the device object.

Why these values. A scan on three S instances (Fig. 1 right; DERIVATIONS §6.6), scored by P(reach), best gap and feasibility, never mean cost: longer is better at every Ω (2000 → 4000 → 5800 ns); 0.75 Ωmax beats full and half power at 4000 and 5800 ns; δf×{0.8,1,1.25} gives 0.915 / 0.932 / 0.884, so the fitted value stays. S_seed02/03, not used in the scan, reach the model optimum in 99 % / 96 % of shots with these settings. Emulators: pulser-simulation (QuTiP) for every S headline row; emu-sv cross-check on S_seed00 (0.96 vs 0.94); an in-house exact state-vector integrator built from Pulser’s own samples for the scans (total-variation distance 0.05–0.07 from QuTiP at 500 shots, i.e. sampling noise); emu-mps for M and L with the bond dimension recorded.

# 5. Results (brief Table 3; full table: results.csv, 79 rows, python -m cvrp.results)

How to read a row. Raw shots, no repair. feas.: fraction of shots that decode to a capacity-feasible grouping. model gap: best feasible shot against the exact model optimum; CVRP gap: that shot routed exactly against C* (proven on S, best known on M/L); both on feasible decodes only, “undefined” if there are none. P(reach C*): shots whose grouping routes to C*, so it includes the decomposition loss. P(gap ≤ 5 %): shots within 5 % of the model optimum; on every S instance and on M_seed00/01 this equals P(reach model optimum) because the next-best feasible grouping is 11–62 % worse, on M_seed02 it does not (six groupings within 5 %; DERIVATIONS Q-M3). fidelity: relative coupling error of the full matrix, lower is better.

| instance | method | feas. | model gap | CVRP gap | P(reach C*) | P(gap≤5%) | fidelity | backend |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S_seed00 | ours | 1.00 | 0.0 % | 0.0 % | 0.94 | 0.94 | 0.26 | qutip |
| S_seed00 | ours, tight A (ablation) | 1.00 | 0.0 % | 0.0 % | 0.03 | 0.03 | 0.31 | qutip |
| S_seed00 | random assignments (one-hot) | 0.63 | 0.0 % | 0.0 % | 0.12 | 0.12 | 0.26 | random-onehot |
| S_seed00 | qubo-solver quantum | 1.00 | 0.0 % | 0.0 % | 0.78 | 0.78 | 0.12 | qs-quantum |
| S_seed00 | qubo-solver tabu | 1.00 | 0.0 % | 0.0 % | 1.00 | 1.00 | undef. | qs-classical |
| S_seed00 | random sampler | 0.04 | 0.0 % | 0.0 % | 0.01 | 0.01 | 0.95 | random |
| S_seed01 | ours | 0.95 | 0.0 % | 0.0 % | 0.95 | 0.95 | 0.21 | qutip |
| S_seed02 | ours | 1.00 | 0.0 % | 0.0 % | 0.99 | 0.99 | 0.31 | qutip |
| S_seed03 | ours | 1.00 | 0.0 % | 15.6 % | 0.01 | 0.96 | 0.29 | qutip |
| S_seed04 | ours | 1.00 | 0.0 % | 21.7 % | 0.02 | 0.89 | 0.32 | qutip |
| M_seed00 | ours (emu-mps) | 0.04 | 67.9 % | 29.9 % | 0.00 | 0.00 | 0.58 | emu-mps χ=32 |
| M_seed00 | qubo-solver quantum | 0.02 | 72.1 % | 35.2 % | 0.00 | 0.00 | 0.09 | qs-quantum χ=32 |
| M_seed01 | ours (emu-mps) | 0.39 | 37.9 % | 9.2 % | 0.00 | 0.00 | 0.08 | emu-mps χ=32 |
| M_seed01 | qubo-solver quantum | 0.37 | 47.5 % | 11.2 % | 0.00 | 0.00 | 0.08 | qs-quantum χ=32 |
| M_seed02 | ours (emu-mps) | 0.45 | 23.0 % | 16.0 % | 0.00 | 0.00 | 0.47 | emu-mps χ=32 |
| M_seed02 | qubo-solver quantum | 0.39 | 34.2 % | 13.6 % | 0.00 | 0.00 | 0.09 | qs-quantum χ=32 |
| L_seed00 | ours (emu-mps) | 0.01 | 140.0 % | 79.7 % | 0.00 | 0.00 | 0.65 | emu-mps χ=32 |
| L_seed01 | ours (emu-mps) | 0.00 | 182.9 % | 75.1 % | 0.00 | 0.00 | 0.65 | emu-mps χ=32 |
| L_seed02 | ours (emu-mps) | 0.00 | undef. | undef. | 0.00 | 0.00 | 0.64 | emu-mps χ=32 |

(generated by python scripts/report_tables.py from results.csv; raw shots only — repair rows are in results.csv under their own label; shots: 500 per emulator row, 2000 for the random sampler, 1 for a classical solver; controls and ablations are shown for S_seed00 only — every other row of every method (random controls, tight-A ablation, tabu, pre-6.7 rows) is in results.csv)

Reference C* (brief §9.3). CPLEX is not importable, so C* is the best of qubo-solver’s simulated annealing on our penalized QUBO and our constrained annealing on the CVRP, each 10 seeded restarts with a 60 s max solve time (settings in instances/*.json). On S the exhaustive dynamic programme is also run and agrees 5/5, so S references are proven; M/L are best known. The QUBO-side annealer reaches C* on 5/5 S and 2/3 M instances but stays 23–34 % above it on all three L (599.9 / 554.6 / 521.7 vs 447.0 / 434.9 / 424.7), so C* is never the QUBO solver alone.

| instance | ρ | C* (best known) | model optimum routed exactly | decomposition gap |
| --- | --- | --- | --- | --- |
| S_seed00 | 0.714 | 262.97 | 262.97 | 0.0 % |
| S_seed01 | 0.857 | 245.02 | 245.02 | 0.0 % |
| S_seed02 | 0.714 | 254.62 | 254.63 | 0.0 % |
| S_seed03 | 0.643 | 197.44 | 228.34 | 15.6 % |
| S_seed04 | 0.714 | 184.42 | 224.49 | 21.7 % |
| M_seed00 | 0.950 | 432.00 | 432.00 | 0.0 % |
| M_seed01 | 0.733 | 368.36 | 368.36 | 0.0 % |
| M_seed02 | 0.700 | 360.86 | 385.62 | 6.9 % |
| L_seed00 | 0.650 | 446.97 | 525.71 | 17.6 % |
| L_seed01 | 0.708 | 434.91 | 470.67 | 8.2 % |
| L_seed02 | 0.600 | 424.66 | 480.17 | 13.1 % |

Model selection, evaluated classically. Each additive objective below is solved exactly with capacity enforced and its groups are then routed exactly. These numbers test objective quality only; the alternatives have not been mapped to the analog register. depot-MST means MSTS+2mini∈Sd0i per group.

| tier | pairwise routed gaps | medoid routed gaps | depot-MST routed gaps |
| --- | --- | --- | --- |
| S | 0.0% / 0.0% / 0.0% / 15.6% / 21.7% | 6.4% / 0.0% / 3.1% / 0.0% / 17.3% | 0.0% / 0.0% / 0.0% / 0.0% / 17.3% |
| M | 0.0% / 0.0% / 6.9% | 0.0% / 0.0% / 6.9% | 4.4% / 2.6% / 7.4% |
| L | 17.6% / 8.2% / 13.1% | 15.6% / 19.8% / 19.8% | 0.0% / 0.0% / 7.5% |

The depot-aware surrogate removes the observed pairwise-objective loss on L_seed00/01 and cuts it on L_seed02, but worsens every M instance. It is a promising L objective, not a universal replacement.

Capacity-binding validation. From seeds 1000–1199 we selected the first eight S instances for which every unconstrained pairwise optimum overloaded a vehicle. Four development cases selected κ=0.6 from {0,.05,.15,.3,.6,1} before the four validation cases were inspected. All values below are raw 500-shot QuTiP runs; intervals are 95 % Wilson intervals conditional on each fixed sampler.

| validation instance | feasibility κ=.15 | feasibility κ=.6 | P(model optimum) .15 / .6 | P(C*) .15 / .6 |
| --- | --- | --- | --- | --- |
| S_seed1052 | 0.03 [0.02,0.05] | 1.00 [0.99,1.00] | 0.00 / 0.00 | 0.00 / 0.05 |
| S_seed1053 | 0.85 [0.81,0.87] | 0.19 [0.16,0.23] | 0.84 / 0.01 | 0.84 / 0.01 |
| S_seed1055 | 0.04 [0.03,0.06] | 1.00 [0.99,1.00] | 0.01 / 0.94 | 0.00 / 0.00 |
| S_seed1057 | 1.00 [0.99,1.00] | 1.00 [0.99,1.00] | 0.00 / 0.26 | 0.70 / 0.71 |

The stronger weight raises mean feasibility on this deliberately difficult holdout from 0.48 to 0.80, but success is instance-dependent: it hurts S_seed1053 and lowers mean P(reach CVRP reference). It is therefore an ablation, not a new headline default. A bounded classical relocation/swap repair raises M feasibility to 0.98–1.00 and can reach 0 % route gap, but matched random one-hot samples do the same. Repair results remain separately labelled in out/two_hour/repair/comparison.csv; raw sampling remains the quantum claim.

Tier S — the device does the work. The model-optimal grouping is returned in 89–99 % of raw shots on all five instances, with 0.95–1.00 of shots feasible. The fair control — random assignment-valid strings through the same decoder — reaches it with probability 2/16 ≈ 0.12 on every instance; uniform random bitstrings 0.7–1.2 %; the generic embedder 3–13 %. qubo-solver’s own pipeline (its embedding and drive, 5940 ns) reaches 0.78 / 0.78 / 0.18 / 0.79 / 0.00 and has a 27 % model gap on S_seed04; its tabu search finds every model optimum classically. Where P(reach C*) is low with a 0 % model gap (S_seed03/04: 0.01 / 0.02) the loss is the decomposition gap (15.6 % / 21.7 %): the closeness objective’s best grouping is not the shortest-route grouping. That is the price of §1, not of the device.

Ablation — the margin in A is physical. Replacing A by its tight provable bound (DERIVATIONS Q-A), same register construction and pulse, moves P(reach model optimum) from 0.94 to 0.03 on S_seed00, 0.95 to 0.52 on S_seed01 and 0.89 to 0.94 on S_seed04: a smaller A shrinks the defect gaps the sweep must resolve.

Tier M — valid groupings, poor ones. With δf bounded by Eq. (4), 98–100 % of shots are one-hot and 4.4 % / 38.8 % / 44.6 % are capacity-feasible on M_seed00/01/02; the best feasible shots are 67.9 / 37.9 / 23.0 % above the model optimum (29.9 / 9.2 / 16.0 % CVRP gap). A uniform random sampler decodes 0 of 2000 shots on every M instance. We enumerated all 59 049 one-hot groupings to locate the loss (DERIVATIONS Q-M3, docs/checks/landscapes.md): (i) the load-balance term leaves the model optimum at QUBO rank 1 on M_seed00/01, but on M_seed02 the QUBO minimum is a feasible grouping 10.2 % above it — exactly what qubo-solver’s tabu search returns — so on M_seed02 about 10 of the 23 points are the λ2 rule; (ii) under the realised register the model optimum sits at rank 10 474 / 12 966 / 4 192 of 59 049, which is the main loss; (iii) the emulated best shots beat the register’s own best feasible string (82.0 / 101.3 / 93.9 %), so the sweep is not the bottleneck. qubo-solver’s independent pipeline lands in the same regime (feasible 0.016 / 0.37 / 0.388, CVRP gap 35.2 / 11.2 / 13.6 %), while its tabu search on the same QUBO reaches 17.6 % / 2.8 % above C* on M_seed01/02: the QUBO is solvable classically; the planar register limits it. Emulator check: M_seed00 needs a bond dimension of only 12, so χ=32 and χ=64 give the same untruncated run (feasible 0.044 vs 0.030, gap 67.9 vs 70.2 %: sampling noise); M_seed01 and L hit the χ=32 cap and are provisional (DERIVATIONS §6.8).

Tier L — almost nothing valid. L_seed00 / L_seed01 / L_seed02 (60 atoms, emu-mps χ=32, 500 shots) produce 0.8 % / 0.4 % / 0 % feasible shots; the first two have 79.7 % / 75.1 % CVRP gaps and the third gap is undefined. All three runs hit the bond-dimension cap and are provisional. The weakest square coupling limits δf to 4.475 rad/µs and compresses final defect gaps, while the model optimum ranks 16 534 / 1 063 / 14 862 among the one-hot energies on L_seed00/01/02. These diagnostics make embedding and dynamics plausible contributors; they do not measure the minimum many-body spectral gap during the pulse.

# 6. What did not work

- Generic embedding of the one-hot graph. Tried least-squares placement of all NK atoms; coupling error 0.95, an infeasible ground state on S_seed04, P(reach) 3–13 %. Cause: the clique forces the zero couplings to be as large as the wanted ones. Replaced by the one-hot-subspace fit (§3): P(reach) 0.89–0.99.

- Unbounded δf on tier M. The first M rows had δf=125.66 rad/µs =2.28 Umin: 0 of 500 shots decodable on M_seed00/02. Cause: two excitations per triangle were favoured. Fixed by Eq. (4); the old rows stay in results.csv under |pre-6.7. Then the triangle version of the bound was applied to tier-L squares: the first L run (δf=38.7) was killed before reporting and the bound rewritten for the weakest pair.

- The one-hot ordering for K=3 and K=4. Opening the M triangles from 5 to 6–10 µm lowers the energy scale without fixing the ordering (optimum rank ≥ 1 279); a K=2 bisection puts it at rank 192 594 / 12 217 (docs/GATE2_OPTIONS.md). We did not find a planar register that orders one-hot groupings by Eq. (1) for K≥3.

- The λ2 rule on M_seed02. The load-balance weight moves the QUBO optimum 10.2 % above the model optimum on one of three M instances (0 % elsewhere). We kept the rule because changing weights after seeing results was ruled out; the effect is quantified in Q-M3.

- Register reproducibility. The polygon optimiser gave two different 30-atom registers from one seed when two runs overlapped (M_seed01, optimum rank 1 114 vs 12 966). Rule adopted: every reported row loads the committed register; one control row was re-measured.

- Classical references. CPLEX is not importable; the S agreement check is brute force vs two annealers. qubo-solver’s default annealer is a single-start descent at T0=5 and misses the S optima, so a stated-parameter annealer is used.

- Weight scan. A 12-point scan of A,λ2 around the rule showed no trend: the embedder’s local minimum moves P(reach) more than the weights do (DERIVATIONS §4.1).

- Classical repair. Capacity-local relocation and swap search can make almost every M/random sample feasible and often reach the reference route. The same behavior on random controls shows that this is classical optimizer work, not evidence of a better quantum sampler.

AI use. Claude and Codex drafted code, derivations, experiments, tables, and prose; the team chose the model and protocol and owns every claim. Algebra is checked by exhaustive S enumeration, device legality by Pulser, numerical tables by scripts, and report claims against saved records. Full accounting is in docs/AI_USE.md.